Recommended and permitted tyre fitments to suit RTA guidelines, without requiring engineering certificate. Based on standard fitment for a MKII Escort. Diameter tolerance based on ± 15mm at standard 578 mm. Wheel width no greater than 26mm on largest option, probably 5½ plus = 6½. Wheel track not to increase more than 25mm on original.

| Wheel Size | Tyre Size | Profile / Ratio | # Outer Dia | Speedo Error | Rim Widths |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Recommended | Min | Max |  |  |
| 13 | 165 | 75 | 578 | 100 |  |  |  |  |  |
| 13 | 175 | 70 | 575 | 99.6 |  |  |  |  |  |
| 13 | 175 | 75 | 593 | 102.6 |  |  |  |  |  |
| 13 | 185 | 65 | 571 | 98.8 |  |  |  |  |  |
| 13 | 185 | 70 | 589 | 102.0 |  |  |  |  |  |
| 13 | 195 | 60 | 564 | 97.7 |  |  |  |  |  |
| 13 | 195 | 65 | 584 | 101.0 |  |  |  |  |  |
| 13 | 205 | 60 | 576 | 99.7 |  |  |  |  |  |
| 13 | 215 | 55 | 567 | 98.1 |  |  |  |  |  |
| 13 | 215 | 60 | 588 | 101.8 |  |  |  |  |  |
| 13 | 225 | 55 | 578 | 100.0 |  |  |  |  |  |
| 14 | 175 | 60 | 566 | 97.9 |  |  |  |  |  |
| 14 | 175 | 65 | 583 | 100.9 |  |  |  |  |  |
| 14 | 185 | 60 | 578 | 100.0 |  |  |  |  |  |
| 14 | 195 | 55 | 570 | 98.7 |  |  |  |  |  |
| 14 | 195 | 60 | 590 | 102.1 |  |  |  |  |  |
| 14 | 205 | 55 | 581 | 100.6 |  |  |  |  |  |
| 14 | 215 | 50 | 571 | 98.8 |  |  |  |  |  |
| 14 | 215 | 55 | 592 | 102.5 |  |  |  |  |  |
| 14 | 225 | 50 | 581 | 100.5 |  |  |  |  |  |
| 15 | 175 | 55 | 574 | 99.3 |  |  |  |  |  |
| 15 | 175 | 60 | 591 | 102.3 |  |  |  |  |  |
| 15 | 185 | 50 | 566 | 98.0 |  |  |  |  |  |
| 15 | 185 | 55 | 585 | 101.2 |  |  |  |  |  |
| 15 | 195 | 50 | 576 | 99.7 |  |  |  |  |  |
| 15 | 205 | 45 | 566 | 97.9 |  |  |  |  |  |
| 15 | 205 | 50 | 586 | 101.4 |  |  |  |  |  |
| 15 | 215 | 45 | 575 | 99.4 |  |  |  |  |  |
| 15 | 225 | 45 | 584 | 101.0 |  |  |  |  |  |
| 16 | 175 | 45 | 564 | 97.6 |  |  |  |  |  |
| 16 | 175 | 50 | 581 | 100.6 |  |  |  |  |  |
| 16 | 185 | 45 | 573 | 99.2 |  |  |  |  |  |
| 16 | 185 | 50 | 591 | 102.4 |  |  |  |  |  |
| 16 | 195 | 45 | 582 | 100.7 |  |  |  |  |  |
| 16 | 205 | 40 | 570 | 98.7 |  |  |  |  |  |
| 16 | 205 | 45 | 591 | 102.3 |  |  |  |  |  |
| 16 | 215 | 40 | 578 | 100.1 |  |  |  |  |  |
| 16 | 225 | 35 | 564 | 97.6 |  |  |  |  |  |
| 16 | 225 | 40 | 586 | 101.5 |  |  |  |  |  |
| 17 | 175 | 40 | 572 | 99.0 |  |  |  |  |  |
| 17 | 175 | 45 | 589 | 102.0 |  |  |  |  |  |
| 17 | 185 | 40 | 580 | 100.4 |  |  |  |  |  |
| 17 | 195 | 35 | 568 | 98.4 |  |  |  |  |  |
| 17 | 195 | 40 | 588 | 101.7 |  |  |  |  |  |
| 17 | 205 | 35 | 575 | 99.6 |  |  |  |  |  |
| 17 | 215 | 35 | 582 | 100.8 |  |  |  |  |  |
| 17 | 225 | 30 | 567 | 98.1 |  |  |  |  |  |
| 17 | 225 | 35 | 589 | 102.0 |  |  |  |  |  |
|  |  |  | 0 | 0.0 |  |  |  |  |  |
| 17 | 205 | 40 | 596 | 103.1 |  |  |  |  |  |
|  |  |  | 0 | 0.0 |  |  |  |  |  |